LAB MANUAL

Basic Router Configuration Using CLI Commands

Lab No.: __________

1. Objective

To learn and practice basic Cisco Router CLI commands for accessing router modes, configuring a hostname and IP address, enabling interfaces, checking configuration, testing connectivity, and saving the configuration.

2. Software Required

• Cisco Packet Tracer
• PC/Laptop

3. Topology

PC1 ---------------- Router R1
       Copper Straight-Through

4. IP Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| PC1 | FastEthernet0 | 192.168.1.2 | 255.255.255.0 |
| R1 | GigabitEthernet0/0 | 192.168.1.1 | 255.255.255.0 |

PC1 Default Gateway: 192.168.1.1

5. Procedure

Step 1: Create the Topology

Open Cisco Packet Tracer. Add one router and one PC. Connect PC1 to Router R1 using a Copper Straight-Through Cable.

Step 2: Configure PC1

Go to PC1 → Desktop → IP Configuration and enter:
IP Address: 192.168.1.2
Subnet Mask: 255.255.255.0
Default Gateway: 192.168.1.1

Step 3: Open Router CLI

Click Router R1 → CLI and press Enter. The prompt will appear as Router>.

Step 4: Enter Privileged EXEC Mode

Router> enable
Router#

Step 5: Enter Global Configuration Mode

Router# configure terminal
Router(config)#

Step 6: Configure Hostname

Router(config)# hostname R1
R1(config)#

Step 7: Configure Router Interface

R1(config)# interface gigabitEthernet 0/0
R1(config-if)# ip address 192.168.1.1 255.255.255.0
R1(config-if)# no shutdown

Step 8: Exit Configuration Mode

R1(config-if)# exit
R1(config)# exit
R1#

Step 9: Check Interface Configuration

R1# show ip interface brief
The GigabitEthernet0/0 interface should show Status = up and Protocol = up.

Step 10: Test Connectivity From Router

R1# ping 192.168.1.2
A successful test normally displays !!!!!.

Step 11: Test Connectivity From PC

Go to PC1 → Desktop → Command Prompt and type:
ping 192.168.1.1

Step 12: Display Running Configuration

R1# show running-config

Step 13: Display Routing Table

R1# show ip route

Step 14: Save Configuration

R1# copy running-config startup-config
Press Enter when asked for the destination filename.

6. Important CLI Commands

| Command | Purpose |
| --- | --- |
| enable | Enter privileged mode |
| configure terminal | Enter global configuration mode |
| hostname R1 | Change router name |
| interface gigabitEthernet 0/0 | Select an interface |
| ip address | Assign an IP address |
| no shutdown | Turn interface ON |
| shutdown | Turn interface OFF |
| show ip interface brief | Check interface status |
| show running-config | Display current configuration |
| show startup-config | Display saved configuration |
| show ip route | Display routing table |
| ping | Test connectivity |
| copy running-config startup-config | Save configuration |
| exit | Exit current mode |

7. Complete CLI Configuration

Router> enable
Router# configure terminal
Router(config)# hostname R1
R1(config)# interface gigabitEthernet 0/0
R1(config-if)# ip address 192.168.1.1 255.255.255.0
R1(config-if)# no shutdown
R1(config-if)# exit
R1(config)# exit
R1# show ip interface brief
R1# ping 192.168.1.2
R1# show running-config
R1# show ip route
R1# copy running-config startup-config

8. Result

The router was successfully configured using Cisco IOS CLI commands. The interface was assigned an IP address, enabled, connectivity was tested using ping, and the configuration was saved successfully.

9. Viva Questions

Q1. What is CLI?

CLI stands for Command-Line Interface. It allows us to configure and manage network devices using commands.

Q2. What is the purpose of enable?

It enters Privileged EXEC mode.

Q3. What does no shutdown do?

It activates an interface.

Q4. Which command checks interface status?

show ip interface brief

Q5. Which command tests connectivity?

ping

Q6. Which command saves the configuration?

copy running-config startup-config

Q7. What does show ip route display?

It displays the router's routing table.

10. Screenshot Evidence

Screenshot 1: Completed Network Topology

[ Paste Screenshot Here ]

Screenshot 2: Router CLI Configuration

[ Paste Screenshot Here ]

Screenshot 3: show ip interface brief

[ Paste Screenshot Here ]

Screenshot 4: Successful Ping Test

[ Paste Screenshot Here ]